______________________________________________________________

UM-SJTU  JOINT  INSTITUTE

PHYSICS  LABORATORY

(VP 241)

______________________________________________________________

LABORATORY  REPORT

EXERCISE 2

THE HALL PROBE: CHARACTERISTICS AND APPLICATIONS

| Name: Weikai Zhou | ID: 518021911039 | Group: 8 |
| --- | --- | --- |
| Name: Yingtao Zou | ID: 518370910069 | Group: 8 |
| Date: 6 December 2019 |  |  |

- Introduction [1]

  - Objectives

- Try to study the principle of the Hall effect and its applications by using a Hall probe.

- Try to verify that the Hall voltage is proportional to the magnetic field

- Try to study the sensitivity of an integrated Hall probe by calculating the magnetic field at the center of a solenoid.

- Try to measure the magnetic field distribution along the axis of the solenoid and compare it with the corresponding theoretical curve.

  - Theoretical background

The phenomenon that when an electric current passes through a sample placed in a magnetic field, electric potential difference proportional to the current and to the magnetic field appears across the material in the direction perpendicular to both the current and the magnetic field is observed by E.H. Hall in 1879. This effect is known as the Hall effect, and since its discovery it has led to many practical applications. The principle of the Hall effect is used in devices for magnetic field measurements as well as in position and motion detectors.

    - Hall effect

If we consider a conducting sheet, which is made of a metal or a semiconductor, placed in a magnetic field so that the plane of the sheet is perpendicular to the direction of the magnetic field B (Figure 1). An electric potential difference between the sides a and b of the sheet will be generated, if the electric current I passes through the sheet in the direction as shown in Figure 1. We call this phenomenon as the Hall effect, and the electric potential difference is called the Hall voltage UH.

Figure 1. The principle of the Hall effect

From a microscopical perspective, the Hall effect is caused by the Lorentz force, which is a force acting on charges moving in a magnetic field. The Lorentz force FB leads to the deflection of the moving charges. These charges accumulate on one side of the sheet and increase the magnitude of the transverse electric field EH (the Hall field). There will be an electric force FE acting upon the charges due to this field. A balance will be eventually reached and UH will stabilize since FB and FE act in opposite direction. The sign of the charge carriers (positive or negative) will determine the sign of UH. Therefore, we can analyze the sign of UH to determine the type of charge carriers in semiconductors.

The Hall voltage is proportional to both the current and the magnitude of the magnetic field, and inversely proportional to the thickness of the sheet d if the external magnetic field is not too strong

UH=RHIBd=KIB                                                        (1)

where RH is the so-called Hall coefficient and K=RH/d=KH/I, where KH is the so-called sensitivity of the Hall element.

    - Integrated hall probe

If we measure Hall voltage with a Hall probe when the sensitivity KH and the current I are fixed, we can easily find the magnitude of the magnetic field. We should amplify the Hall voltage before the measurement since it is usually very small.

Silicon can be used to design both the Hall probe and the integrated circuits, so it is convenient to arrange the Hall probe and the electric circuits into a single device. And we usually call such a device an integrated Hall probe.

The integrated Hall probe SS495A consists of a Hall sensor, an amplifier, and a voltage compensator (Figure 2). We can read the output voltage U ignoring the residual voltage. The working voltage US=5 V, and the output voltage U0 is approximately 2.5 V when the magnetic field is zero. The relation between the output voltage U and the magnetic field is

B=U-U0KH                                                               (2)

Figure 2. The integrated Hall probe SS495A (left). The relation between the

output voltage U and the magnitude of the magnetic field B (right).

    - Magnetic field distribution inside a solenoid

We use the following formula to calculate the magnetic field distribution on the axis of a single layer solenoid

Bx=μ0NLIML+2x2D2+L+2x212+L-2x2D2+L-2x212=CxIM        (3)

where N is the number of turns of the solenoid, L is its length, IM is the current through the solenoid wire, and D is the solenoid’s diameter. The magnetic permeability of vacuum is μ0=4π×10-7 H/m.

We use the solenoid with ten layers for this experiment. The magnetic field Bx for each layer can be calculated using Eq. (3). We can add contributions of all layers to find the net magnetic on the axis of the solenoid. The theoretical value of the magnetic field inside the solenoid with IM=0.1 A is given in Table 1.

| x [cm] | B [mT] | x [cm] | B [mT] |
| --- | --- | --- | --- |
| ±0.0 | 1.4366 | ±8.0 | 1.4057 |
| ±1.0 | 1.4363 | ±9.0 | 1.3856 |
| ±2.0 | 1.4356 | ±10.0 | 1.3478 |
| ±3.0 | 1.4343 | ±11.0 | 1.2685 |
| ±4.0 | 1.4323 | ±11.5 | 1.1963 |
| ±5.0 | 1.4292 | ±12.0 | 1.0863 |
| ±6.0 | 1.4245 | ±12.5 | 0.9261 |
| ±7.0 | 1.4173 | ±13.0 | 0.7233 |

Table 1. Theoretical value of the magnetic field inside the solenoid

    - Study of the geomagnetic field with a Hall probe (optional)

The geomagnetic field is the magnetic associated with the Earth. The geomagnetic field lines are shown schematically in Figure 3. The Earth’s magnetic field is similar to that of a bar magnet tilted about 11.5 degrees from the spin axis of the Earth.

Figure 3. Magnetic field of the Earth

The figure below (Figure 4) shows the geomagnetic field distribution of China in 1970. We can see that the magnetic inclination is about 44.5°and the magnitude of the magnetic field in Shanghai is about 48000 nT.

Figure 4. Geomagnetic inclination in China, 1970 (left). The

magnitude of the geomagnetic field in China, 1970 (right).

- Apparatus [1]

  - Experimental setup

The experimental setup shown in Figure 5 consists of an integrated Hall probe SS495A (Figure 6) with KH=31.25±1.25 V/T (at the working voltage 5 V) or KH=3.125±0.125 mV/G, a solenoid, a power supply, a voltmeter, a DC voltage divider, and a set of connecting wires.

Figure 5. Measurement setup

Figure 6. Integrated Hall probe SS495A

  - Precision or uncertainty

| US (voltage source) | 0.5% V |
| --- | --- |
| U0 & U (voltimeter) | 0.05%+6×10-3 or 6×10-4 [V] |
| IM (current source) | 2% [mA] |
| x (distance) | 0.05 [cm] |

Table 2. precision or uncertainty

- Measurement procedure [1]

  - Relation between sensitivity KH and working voltage US

    - We place the integrated Hall probe at the center of the solenoid. Then, we set the working voltage at 5 V and measure the output voltage U0 (IM = 0) and U (IM = 250 mA). We take the theoretical value of B (x = 0) from Table 1 and calculate the sensitivity of the probe KH by using Eq. (2).

    - Then, we measure KH for different values of US (form 2.8 V to 10 V). We should calculate KH/US and plot the curve KH/US vs. US.

  - Relation between output voltage U and magnetic field B

    - We should connect the 2.4~2.6 V output terminal of the DC voltage divider and the negative port to the voltmeter with B=0, US=5 V. then, we should adjust the voltage until U0=0.

    - We should place the integrated Hall probe at the center of the solenoid and measure the output voltage U for different values of IM ranging from 0 to 500 mA, with intervals of 50 mA.

    - We should explain the relation between B (x = 0) and the Hall voltage UH. We should pay attention to the fact that the output voltage U is the amplified signal from UH. The theoretical value of B (x = 0) can be found from Table 1.

    - We should plot the curve U vs. B and find the sensitivity KH by a linear fit (use a computer). We should compare the value we obtained with the theoretical value in given in the Apparatus section and the value we have found in the first part.

  - Magnetic field distribution inside the solenoid

    - We should measure the magnetic field distribution along the axis of the solenoid for IM = 250 mA, record the output voltage U and the corresponding position x. Then we should find B = B(x). (We can use the value of KH found in the previous part of the experiment).

    - We should use a computer to plot the theoretical and the experimental curve showing the magnetic field distribution inside the solenoid. We should use dots for the data measured and a solid line for the theoretical curve. The origin of the plot should be at the center of the solenoid.

  - Measurement of the geomagnetic field (optional)

    - We should use the integrated Hall probe to measure the magnitude and the direction of the geomagnetic field.

  - Cautions

    - We should make sure that the V + port and the V − port of the integrated Hall probe are connected correctly, otherwise the probe will get damaged.

    - After turning the power supply on, we should wait for 5 minutes before starting measurements, in order to let the power supply temperature reaches a steady state.

    - Working voltage US must be lower than 10 V for measurements described in part 3.1..

    - We should set the output voltage and the current to zero before turning off the power supply.

    - We should turn off the power supply before disassembling the equipment.

- Results

  - Relation between sensitivity KH and working voltage US

The data we get are listed below (Table 3&4).

| US[V]±0.5%[V] | U0 (IM = 0) [V] ±0.05%+6×10-3/10-4 [V] | U (IM = 250 mA) [V] ±0.05%+6×10-3/10-4 [V] |
| --- | --- | --- |
| 5.00 | 2.513 | 2.634 |

Table 3. Data for U0 and U with US=5 V

|  | US[V]±0.5%[V] | U0 (IM = 0) [V] ±0.05%+6×10-3/10-4 [V] | U (IM = 250 mA) [V] ±0.05%+6×10-3/10-4 [V] |
| --- | --- | --- | --- |
| 1 | 2.80 | 1.3970 | 1.4677 |
| 2 | 3.20 | 1.5983 | 1.6796 |
| 3 | 3.60 | 1.8050 | 1.8964 |
| 4 | 4.00 | 2.0053 | 2.1061 |
| 5 | 4.40 | 2.2103 | 2.317 |
| 6 | 4.80 | 2.410 | 2.525 |
| 7 | 5.20 | 2.614 | 2.738 |
| 8 | 5.60 | 2.815 | 2.942 |
| 9 | 6.00 | 3.013 | 3.148 |
| 10 | 6.40 | 3.213 | 3.350 |
| 11 | 6.80 | 3.406 | 3.546 |
| 12 | 7.20 | 3.601 | 3.745 |
| 13 | 7.60 | 3.791 | 3.936 |
| 14 | 8.00 | 3.980 | 4.127 |
| 15 | 8.40 | 4.167 | 4.312 |
| 16 | 8.80 | 4.355 | 4.501 |
| 17 | 9.40 | 4.632 | 4.784 |
| 18 | 10.00 | 4.915 | 5.066 |

Table 4. Data for U0 and U with different US

From Table 1, we can get that the theoretical value of Bx=0=1.4366 mT=1.4366×10-3 T with IM_t=0.1 A. While in our experiment, IM_e=250 mA=0.25 A, we can calculate B in this part as follows

B=Bx=0×IM_eIM_t=0.00359±0.00007 T

    - Data for U0 and U with US=5 V

For the data recorded in Table 3, we can use Eq. (2) to get that

KH=U-U0B=2.634-2.5130.00359=34±3 V/T

with relative uncertainty as 334×100%=8.8%. Although it is larger than 5%, we may still think this part is relatively successful because it is still within 10%. The reasons may be the precision of the device. For example, the unit of IM is [A] when we adjust it on the device, while the unity required is [mA], which will increase error. If we can use better device, the result will be better. Besides, we only measure the data for once. If we measure more times, it will be better.

    - Data for U0 and U with different US

Using the first measurement as an example, we can calculate as follows

KH=U-U0B=1.4677-1.39700.00359=19.7±0.7 V/T

KHUS=U-U0BUS=U-U0BUS=1.4677-1.39700.00359×2.80=7.0±0.2 T-1

The whole table is listed below for the answer of the rest calculation (Table 5).

|  | US [V] | uUS [V] | KH [V/T] | uKH [V/T] | KHUS [T-1] | uKHUS [T-1] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2.80 | 0.014 | 19.7 | 0.7 | 7.0 | 0.2 |
| 2 | 3.20 | 0.016 | 22.6 | 0.7 | 7.1 | 0.2 |
| 3 | 3.60 | 0.018 | 25.4 | 0.8 | 7.1 | 0.2 |
| 4 | 4.00 | 0.02 | 28.1 | 0.9 | 7.0 | 0.2 |
| 5 | 4.40 | 0.02 | 30 | 2 | 6.8 | 0.5 |
| 6 | 4.80 | 0.02 | 32 | 3 | 6.7 | 0.6 |
| 7 | 5.20 | 0.03 | 35 | 3 | 6.6 | 0.6 |
| 8 | 5.60 | 0.03 | 35 | 3 | 6.3 | 0.5 |
| 9 | 6.00 | 0.03 | 38 | 3 | 6.3 | 0.5 |
| 10 | 6.40 | 0.03 | 38 | 3 | 6.0 | 0.5 |
| 11 | 6.80 | 0.03 | 39 | 3 | 5.7 | 0.5 |
| 12 | 7.20 | 0.04 | 40 | 3 | 5.6 | 0.4 |
| 13 | 7.60 | 0.04 | 40 | 3 | 5.3 | 0.4 |
| 14 | 8.00 | 0.04 | 41 | 3 | 5.1 | 0.4 |
| 15 | 8.40 | 0.04 | 40 | 3 | 4.8 | 0.4 |
| 16 | 8.80 | 0.04 | 41 | 3 | 4.6 | 0.4 |
| 17 | 9.40 | 0.05 | 42 | 3 | 4.5 | 0.4 |
| 18 | 10.00 | 0.05 | 42 | 3 | 4.2 | 0.3 |

Table 5. Uncertainty for US and KHUS

Then, based on this table, we can plot the curve for KHUS vs. US with origin (Figure 7).

Figure 7. KHUS vs. US

Theoretically, KH should be proportional to US, which means the graph we get should be like a horizontal line. In this figure, we can find that it tends to decline. The reasons for this may be due to the internal resistance of the wire or the precision of the device. But generally speaking, it still maintains in some range, which can prove that KH should be proportional to US.

  - Relation between output voltage U and magnetic field B

The data we get are listed below (Table 6).

|  | IM mA±2% [mA] | U mV±0.05%+6×10-3/10-4 [V] |
| --- | --- | --- |
| 1 | 0 | 0.00 |
| 2 | 50 | 27.35 |
| 3 | 100 | 49.86 |
| 4 | 150 | 73.21 |
| 5 | 200 | 98.44 |
| 6 | 250 | 119.66 |
| 7 | 300 | 143.82 |
| 8 | 350 | 165.28 |
| 9 | 400 | 186.52 |
| 10 | 450 | 209.82 |
| 11 | 500 | 233.4 |

Table 6. Measurement data for the IM vs. U relation

Using the second measurement as an example, we can calculate the magnetic field as follows

B2=B(x=0)IM_t×IM_2=1.4366×10-30.1×50×10-3=0.000718±0.000014 T

The whole table is listed below for the answer of the rest calculation (Table 7).

|  | IM [A] | uIM [A] | U [V] | uU [V] | B [T] | uB [T] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0.00000 | 0.0006 | 0 | 0 |
| 2 | 0.050 | 0.0010 | 0.02735 | 0.0006 | 0.000718 | 0.000014 |
| 3 | 0.100 | 0.002 | 0.04986 | 0.0006 | 0.00144 | 0.00003 |
| 4 | 0.150 | 0.003 | 0.07321 | 0.0006 | 0.00215 | 0.00004 |
| 5 | 0.200 | 0.004 | 0.09844 | 0.0006 | 0.00287 | 0.00006 |
| 6 | 0.250 | 0.005 | 0.11966 | 0.0007 | 0.00359 | 0.00007 |
| 7 | 0.300 | 0.006 | 0.14382 | 0.0007 | 0.00431 | 0.00009 |
| 8 | 0.350 | 0.007 | 0.16528 | 0.0007 | 0.00503 | 0.00010 |
| 9 | 0.400 | 0.008 | 0.18652 | 0.0007 | 0.00575 | 0.00011 |
| 10 | 0.450 | 0.009 | 0.20982 | 0.0007 | 0.00646 | 0.00013 |
| 11 | 0.500 | 0.010 | 0.2334 | 0.0007 | 0.00718 | 0.00014 |

Table 7. Calculated uncertainty for IM, U and B

Based on this table, we can plot the relation of U vs. B with Origin (Figure 8).

Figure 8. Linear fit of U vs. B

Form this figure, we know that U and B are linearly dependent since r2=0.99947 and Pearson’s r = 0.99973, which are close to 1. And U=a×B+b, where a=32.15732 and b=0.003366.

Therefore, we can get that

KH=32.2±0.6 V/T

Theoretically, KH=31.25±1.25 V/T, and the value we get in the previous part is KH=34±3 V/T. So, we can calculate the relative error respectively as

32.2-31.2532.2×100%=2.95%

34-32.232.2×100%=5.59%

We can see that the relative error is small, so we may assume that this part is quite successful.

  - Magnetic field distribution inside the solenoid

The data we get are listed below (Table 8).

|  | x [cm] ±0.05 [cm] | U mV±0.05% +6×10-3/10-4 [V] |  | x [cm] ±0.05 [cm] | U mV±0.05% +6×10-3/10-4 [V] |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.00 | 10.91 | 27 | 17.00 | 119.62 |
| 2 | 0.50 | 14.04 | 28 | 18.00 | 119.59 |
| 3 | 1.00 | 19.27 | 29 | 19.00 | 119.31 |
| 4 | 1.50 | 26.75 | 30 | 20.00 | 119.29 |
| 5 | 2.00 | 38.60 | 31 | 21.00 | 119.05 |
| 6 | 2.50 | 53.68 | 32 | 22.00 | 118.36 |
| 7 | 3.00 | 70.97 | 33 | 23.00 | 117.63 |
| 8 | 3.50 | 85.49 | 34 | 24.00 | 116.15 |
| 9 | 4.00 | 96.32 | 35 | 25.00 | 114.10 |
| 10 | 4.50 | 103.43 | 36 | 25.50 | 112.51 |
| 11 | 5.00 | 107.81 | 37 | 26.00 | 110.44 |
| 12 | 0.25 | 12.33 | 38 | 26.50 | 106.80 |
| 13 | 0.75 | 16.49 | 39 | 27.00 | 101.31 |
| 14 | 1.25 | 22.74 | 40 | 27.50 | 93.87 |
| 15 | 1.75 | 30.81 | 41 | 27.75 | 88.43 |
| 16 | 6.00 | 113.23 | 42 | 28.00 | 81.80 |
| 17 | 7.00 | 115.77 | 43 | 28.25 | 72.80 |
| 18 | 8.00 | 117.29 | 44 | 28.50 | 66.13 |
| 19 | 9.00 | 118.12 | 45 | 28.75 | 56.73 |
| 20 | 10.00 | 118.80 | 46 | 29.00 | 47.58 |
| 21 | 11.00 | 119.18 | 47 | 29.25 | 40.91 |
| 22 | 12.00 | 119.31 | 48 | 29.50 | 34.29 |
| 23 | 13.00 | 119.43 | 49 | 29.75 | 29.03 |
| 24 | 14.00 | 119.30 | 50 | 30.00 | 24.11 |
| 25 | 15.00 | 119.50 | 51 | 2.25 | 46.49 |
| 26 | 16.00 | 119.71 | 52 | 2.75 | 62.37 |

Table 8. Data for the U vs. x relation

Using the first measurement as an example, we can calculate that

B=UKH=10.91×10-332.2=0.00034±0.00002 T

The whole table is listed below for the answer of the rest calculation (Table 9).

|  | x [cm] | ux [cm] | U V | uU [V] | B [T] | uB [T] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.00 | 0.05 | 0.01091 | 0.0006 | 0.00034 | 0.00002 |
| 2 | 0.50 | 0.05 | 0.01404 | 0.0006 | 0.00044 | 0.00002 |
| 3 | 1.00 | 0.05 | 0.01927 | 0.0006 | 0.00060 | 0.00002 |
| 4 | 1.50 | 0.05 | 0.02675 | 0.0006 | 0.00083 | 0.00002 |
| 5 | 2.00 | 0.05 | 0.03860 | 0.0006 | 0.00120 | 0.00003 |
| 6 | 2.50 | 0.05 | 0.05368 | 0.0006 | 0.00167 | 0.00004 |
| 7 | 3.00 | 0.05 | 0.07097 | 0.0006 | 0.00220 | 0.00005 |
| 8 | 3.50 | 0.05 | 0.08549 | 0.0006 | 0.00265 | 0.00005 |
| 9 | 4.00 | 0.05 | 0.09632 | 0.0006 | 0.00299 | 0.00006 |
| 10 | 4.50 | 0.05 | 0.10343 | 0.0007 | 0.00321 | 0.00006 |
| 11 | 5.00 | 0.05 | 0.10781 | 0.0007 | 0.00335 | 0.00007 |
| 12 | 0.25 | 0.05 | 0.01233 | 0.0006 | 0.00038 | 0.00002 |
| 13 | 0.75 | 0.05 | 0.01649 | 0.0006 | 0.00051 | 0.00002 |
| 14 | 1.25 | 0.05 | 0.02274 | 0.0006 | 0.00071 | 0.00002 |
| 15 | 1.75 | 0.05 | 0.03081 | 0.0006 | 0.00096 | 0.00003 |
| 16 | 6.00 | 0.05 | 0.11323 | 0.0007 | 0.00352 | 0.00007 |
| 17 | 7.00 | 0.05 | 0.11577 | 0.0007 | 0.00360 | 0.00007 |
| 18 | 8.00 | 0.05 | 0.11729 | 0.0007 | 0.00364 | 0.00007 |
| 19 | 9.00 | 0.05 | 0.11812 | 0.0007 | 0.00367 | 0.00007 |
| 20 | 10.00 | 0.05 | 0.11880 | 0.0007 | 0.00369 | 0.00007 |
| 21 | 11.00 | 0.05 | 0.11918 | 0.0007 | 0.00370 | 0.00007 |
| 22 | 12.00 | 0.05 | 0.11931 | 0.0007 | 0.00371 | 0.00007 |
| 23 | 13.00 | 0.05 | 0.11943 | 0.0007 | 0.00371 | 0.00007 |
| 24 | 14.00 | 0.05 | 0.11930 | 0.0007 | 0.00370 | 0.00007 |
| 25 | 15.00 | 0.05 | 0.11950 | 0.0007 | 0.00371 | 0.00007 |
| 26 | 16.00 | 0.05 | 0.11971 | 0.0007 | 0.00372 | 0.00007 |
| 27 | 17.00 | 0.05 | 0.11962 | 0.0007 | 0.00371 | 0.00007 |
| 28 | 18.00 | 0.05 | 0.11959 | 0.0007 | 0.00371 | 0.00007 |
| 29 | 19.00 | 0.05 | 0.11931 | 0.0007 | 0.00371 | 0.00007 |
| 30 | 20.00 | 0.05 | 0.11929 | 0.0007 | 0.00370 | 0.00007 |
| 31 | 21.00 | 0.05 | 0.11905 | 0.0007 | 0.00370 | 0.00007 |
| 32 | 22.00 | 0.05 | 0.11836 | 0.0007 | 0.00368 | 0.00007 |
| 33 | 23.00 | 0.05 | 0.11763 | 0.0007 | 0.00365 | 0.00007 |
| 34 | 24.00 | 0.05 | 0.11615 | 0.0007 | 0.00361 | 0.00007 |
| 35 | 25.00 | 0.05 | 0.11410 | 0.0007 | 0.00354 | 0.00007 |
| 36 | 25.50 | 0.05 | 0.11251 | 0.0007 | 0.00349 | 0.00007 |
| 37 | 26.00 | 0.05 | 0.11044 | 0.0007 | 0.00343 | 0.00007 |
| 38 | 26.50 | 0.05 | 0.10680 | 0.0007 | 0.00332 | 0.00007 |
| 39 | 27.00 | 0.05 | 0.10131 | 0.0007 | 0.00315 | 0.00006 |
| 40 | 27.50 | 0.05 | 0.09387 | 0.0006 | 0.00292 | 0.00006 |
| 41 | 27.75 | 0.05 | 0.08843 | 0.0006 | 0.00275 | 0.00005 |
| 42 | 28.00 | 0.05 | 0.08180 | 0.0006 | 0.00254 | 0.00005 |
| 43 | 28.25 | 0.05 | 0.07280 | 0.0006 | 0.00226 | 0.00005 |
| 44 | 28.50 | 0.05 | 0.06613 | 0.0006 | 0.00205 | 0.00004 |
| 45 | 28.75 | 0.05 | 0.05673 | 0.0006 | 0.00176 | 0.00004 |
| 46 | 29.00 | 0.05 | 0.04758 | 0.0006 | 0.00148 | 0.00003 |
| 47 | 29.25 | 0.05 | 0.04091 | 0.0006 | 0.00127 | 0.00003 |
| 48 | 29.50 | 0.05 | 0.03429 | 0.0006 | 0.00106 | 0.00003 |
| 49 | 29.75 | 0.05 | 0.02903 | 0.0006 | 0.00090 | 0.00003 |
| 50 | 30.00 | 0.05 | 0.02411 | 0.0006 | 0.00075 | 0.00002 |
| 51 | 2.25 | 0.05 | 0.04649 | 0.0006 | 0.00144 | 0.00003 |
| 52 | 2.75 | 0.05 | 0.06237 | 0.0006 | 0.00194 | 0.00004 |

Table 9. Calculated uncertainty for x, U, B

For the theoretical value, we can calculate them as

B=B(x)IM_t×IM=1.4366×10-30.1×0.25=0.00359±0.00007 T

The whole table is listed below for the answer of the rest calculation (Table 10).

| x [cm] | Bx [mT] | B [T] | uB [T] |
| --- | --- | --- | --- |
| 0.0 | 0.7233 | 0.00181 | 0.00004 |
| 0.5 | 0.9261 | 0.00232 | 0.00005 |
| 1.0 | 1.0863 | 0.00272 | 0.00005 |
| 1.5 | 1.1963 | 0.00299 | 0.00006 |
| 2.0 | 1.2685 | 0.00317 | 0.00006 |
| 3.0 | 1.3478 | 0.00337 | 0.00007 |
| 4.0 | 1.3856 | 0.00346 | 0.00007 |
| 5.0 | 1.4057 | 0.00351 | 0.00007 |
| 6.0 | 1.4173 | 0.00354 | 0.00007 |
| 7.0 | 1.4245 | 0.00356 | 0.00007 |
| 8.0 | 1.4292 | 0.00357 | 0.00007 |
| 9.0 | 1.4323 | 0.00358 | 0.00007 |
| 10.0 | 1.4343 | 0.00359 | 0.00007 |
| 11.0 | 1.4356 | 0.00359 | 0.00007 |
| 12.0 | 1.4363 | 0.00359 | 0.00007 |
| 13.0 | 1.4366 | 0.00359 | 0.00007 |
| 14.0 | 1.4363 | 0.00359 | 0.00007 |
| 15.0 | 1.4356 | 0.00359 | 0.00007 |
| 16.0 | 1.4343 | 0.00359 | 0.00007 |
| 17.0 | 1.4323 | 0.00358 | 0.00007 |
| 18.0 | 1.4292 | 0.00357 | 0.00007 |
| 19.0 | 1.4245 | 0.00356 | 0.00007 |
| 20.0 | 1.4173 | 0.00354 | 0.00007 |
| 21.0 | 1.4057 | 0.00351 | 0.00007 |
| 22.0 | 1.3856 | 0.00346 | 0.00007 |
| 23.0 | 1.3478 | 0.00337 | 0.00007 |
| 24.0 | 1.2685 | 0.00317 | 0.00006 |
| 24.5 | 1.1963 | 0.00299 | 0.00006 |
| 25.0 | 1.0863 | 0.00272 | 0.00005 |
| 25.5 | 0.9261 | 0.00232 | 0.00005 |
| 26.0 | 0.7233 | 0.00181 | 0.00004 |

Table 10. calculated uncertainty for B

Based on Table 9 and 10, we can get the figure below (Figure 9)

Figure 9. The theoretical and experimental B vs. x

From this figure, we know that the magnetic field usually is its largest at the center of the solenoid while becomes smaller at the end of the solenoid. Generally, the trend of the experimental one and theoretical one are similar. But there are still some differences. For example, the experimental one shifted to the right compared with the theoretical one. We may think that the actual center of the solenoid is not at 13.0 cm but around 16.0 cm according to the data we have, and we can repeat the experiment for many times to get the position. Besides, during the experiment, we find that the readings on the meter are not stable, this may be because of the precision of the device.

- Conclusion [1]

  - Relation between sensitivity KH and working voltage US

In this part, we first calculate the magnetic field for IM=250 mA
B=0.00359±0.00007 T

Then, for US=5 V, we calculate
KH=U-U0B=2.634-2.5130.00359=34±3 V/T

with relative uncertainty as 334×100%=8.8%. Although it is larger than 5%, we may still think this part is relatively successful because it is still within 10%. The reasons may be the precision of the device. For example, the unit of IM is [A] when we adjust it on the device, while the unity required is [mA], which will increase error. If we can use better device, the result will be better. Besides, we only measure the data for once. If we measure more times, it will be better.

For varying US, we first calculate KHUS, and then plot the curve for KHUS vs. US. In this figure, we can find that it tends to decline. Theoretically, KH should be proportional to US, which means the graph we get should be like a horizontal line. The reasons for this may be due to the internal resistance of the wire or the precision of the device. But generally speaking, it still maintains in some range, which can prove that KH should be proportional to US.

  - Relation between output voltage U and magnetic field B

In this part, we first calculate magnetic field, the plot the relation of U vs. B with Origin.

Form the figure, we know that U and B are linearly dependent since r2=0.99947 and Pearson’s r = 0.99973, which are close to 1. And U=a×B+b, where a=32.15732 and b=0.003366.

Therefore, we can get that

KH=32.2±0.6 V/T

Theoretically, KH=31.25±1.25 V/T, and the value we get in the previous part is KH=34±3 V/T. So, we can calculate the relative error respectively as

32.2-31.2532.2×100%=2.95%

34-32.232.2×100%=5.59%

We can see that the relative error is small, so we may assume that this part is quite successful.

  - Magnetic field distribution inside the solenoid

In this part, we first calculate magnetic field of experimental and theoretical one. Then we plot B vs. x. From the figure, we know that the magnetic field usually is its largest at the center of the solenoid while becomes smaller at the end of the solenoid. Generally, the trend of the experimental one and theoretical one are similar. But there are still some differences. For example, the experimental one shifted to the right compared with the theoretical one. We may think that the actual center of the solenoid is not at 13.0 cm but around 16.0 cm according to the data we have, and we can repeat the experiment for many times to get the position. Besides, during the experiment, we find that the readings on the meter are not stable, this may be because of the precision of the device.

- References

[1] Exercise 2 - lab manual [rev 3.9], UM-JI SJTU. Edited by Qin Tian, Wang Zhiyu, Lin Yiqiao, Bao Yufan, Mateusz Krzyzosiak.

[2] Uncertainty analysis handbook, UM-JI SJTU.

- Uncertainty analysis [2]

A.1. Uncertainty for relation between sensitivity KH and working voltage US

Since Bx=0=1.4366×10-3 T and IM_t=0.1 A are theoretical value, we can assume that uBx=0=0 and uIM_t=0. For IM_e=250 mA, according to the precision or uncertainty of the device, we can calculate its uncertainty as follows

uIM_e=250×2%=5 mA=0.005 A

Then, according to the B=Bx=0×IM_eIM_t, we can calculate the uncertainty for B as

uB=∂B∂Bx=0∙uBx=02+∂B∂IM_t∙uIM_t2+∂B∂IM_e∙uIM_e2=Bx=0IM_t∙uIM_e2=1.4366×10-30.1∙0.0052=0.00007 T

A.1.1. Data for U0 and U with US=5 V

Then, according to the KH=U-U0B, we can calculate the uncertainty for KH as

uKH=∂KH∂U∙uU2+∂KH∂U0∙uU02+∂KH∂B∙uB2=uUB2+uU0B2+(U-U0)∙uBB22

For uU and uU0, according to the precision or uncertainty of the device, we can calculate as follows

uU=2.634×0.05%+6×10-3=0.007 V

uU0=2.513×0.05%+6×10-3=0.007 V

Then, we can get uKH as

uKH=0.0070.003592+0.0070.003592+(2.634-2.513)∙0.000070.0035922=3 V/T

A.1.1. Data for U0 and U with different US

Using the first measurement as an example. For uUS, uU and uU0, according to the precision or uncertainty of the device, we can calculate as follows

uUS=2.80×0.5%=0.014 V

uU0=1.3970×0.05%+6×10-4=0.0013 V

uU=1.4677×0.05%+6×10-4=0.0013 V

According to the KH=U-U0B, we can calculate the uncertainty for KH as

uKH=∂KH∂U∙uU2+∂KH∂U0∙uU02+∂KH∂B∙uB2=uUB2+uU0B2+U-U0∙uBB22=0.00130.003592+0.00130.003592+1.4677-1.3970×0.000070.0035922=0.7 V/T

According to the equation that KHUS=U-U0BUS, we can calculate the uncertainty as

uKHUS=∂KHUS∂U∙uU2+∂KHUS∂U0∙uU02+∂KHUS∂B∙uB2+∂KHUS∂US∙uUS2=uUBUS2+uU0BUS2+U-U0B2US∙uB2+U-U0BUS2∙uUS2=0.00130.00359×2.802+0.00130.00359×2.802+1.4677-1.39700.003592×2.80∙0.000072+1.4677-1.39700.00359×2.802∙0.0142=0.2 T-1

The table below (Table 11) lists the uncertainty calculated for rest data.

|  | US [V] | uUS [V] | U0 [V] | uU0 [V] | U [V] | uU [V] | KH [V/T] | uKH [V/T] | KHUS [T-1] | uKHUS [T-1] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2.80 | 0.014 | 1.3970 | 0.0013 | 1.4677 | 0.0013 | 19.7 | 0.7 | 7.0 | 0.2 |
| 2 | 3.20 | 0.016 | 1.5983 | 0.0014 | 1.6796 | 0.0014 | 22.6 | 0.7 | 7.1 | 0.2 |
| 3 | 3.60 | 0.018 | 1.8050 | 0.0015 | 1.8964 | 0.0015 | 25.4 | 0.8 | 7.1 | 0.2 |
| 4 | 4.00 | 0.02 | 2.0053 | 0.0016 | 2.1061 | 0.0017 | 28.1 | 0.9 | 7.0 | 0.2 |
| 5 | 4.40 | 0.02 | 2.2103 | 0.0017 | 2.317 | 0.007 | 30 | 2 | 6.8 | 0.5 |
| 6 | 4.80 | 0.02 | 2.410 | 0.007 | 2.525 | 0.007 | 32 | 3 | 6.7 | 0.6 |
| 7 | 5.20 | 0.03 | 2.614 | 0.007 | 2.738 | 0.007 | 35 | 3 | 6.6 | 0.6 |
| 8 | 5.60 | 0.03 | 2.815 | 0.007 | 2.942 | 0.007 | 35 | 3 | 6.3 | 0.5 |
| 9 | 6.00 | 0.03 | 3.013 | 0.008 | 3.148 | 0.008 | 38 | 3 | 6.3 | 0.5 |
| 10 | 6.40 | 0.03 | 3.213 | 0.008 | 3.350 | 0.008 | 38 | 3 | 6.0 | 0.5 |
| 11 | 6.80 | 0.03 | 3.406 | 0.008 | 3.546 | 0.008 | 39 | 3 | 5.7 | 0.5 |
| 12 | 7.20 | 0.04 | 3.601 | 0.008 | 3.745 | 0.008 | 40 | 3 | 5.6 | 0.4 |
| 13 | 7.60 | 0.04 | 3.791 | 0.008 | 3.936 | 0.008 | 40 | 3 | 5.3 | 0.4 |
| 14 | 8.00 | 0.04 | 3.980 | 0.008 | 4.127 | 0.008 | 41 | 3 | 5.1 | 0.4 |
| 15 | 8.40 | 0.04 | 4.167 | 0.008 | 4.312 | 0.008 | 40 | 3 | 4.8 | 0.4 |
| 16 | 8.80 | 0.04 | 4.355 | 0.008 | 4.501 | 0.008 | 41 | 3 | 4.6 | 0.4 |
| 17 | 9.40 | 0.05 | 4.632 | 0.008 | 4.784 | 0.008 | 42 | 3 | 4.5 | 0.4 |
| 18 | 10.00 | 0.05 | 4.915 | 0.008 | 5.066 | 0.009 | 42 | 3 | 4.2 | 0.3 |

Table 11. Calculated uncertainty for US, U0, U, KH and KHUS

A.2. Uncertainty for relation between output voltage U and magnetic field B

Using the second measurement as an example, we can calculate the uncertainty for IM and U as follows

uIM=50×2%=1.0 mA=0.0010 A

uU=27.35×10-3×0.05%+6×10-4=0.0006 V

Since Bx=0=1.4366×10-3 T and IM_t=0.1 A are theoretical value, we can assume that uBx=0=0 and uIM_t=0. According to the equation that B2=B(x=0)IM_t×IM_2, we can calculate the uncertainty for B2 as follows

uB2=∂B2∂Bx=0∙uBx=02+∂B2∂IM_t∙uIM_t2+∂B2∂IM_2∙uIM_22=B(x=0)IM_t×uIM_22=1.4366×10-30.1×0.001=0.000014 T

The table below (Table 12) lists the uncertainty calculated for rest data.

|  | IM [A] | uIM [A] | U [V] | uU [V] | B [T] | uB [T] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0.00000 | 0.0006 | 0 | 0 |
| 2 | 0.050 | 0.0010 | 0.02735 | 0.0006 | 0.000718 | 0.000014 |
| 3 | 0.100 | 0.002 | 0.04986 | 0.0006 | 0.00144 | 0.00003 |
| 4 | 0.150 | 0.003 | 0.07321 | 0.0006 | 0.00215 | 0.00004 |
| 5 | 0.200 | 0.004 | 0.09844 | 0.0006 | 0.00287 | 0.00006 |
| 6 | 0.250 | 0.005 | 0.11966 | 0.0007 | 0.00359 | 0.00007 |
| 7 | 0.300 | 0.006 | 0.14382 | 0.0007 | 0.00431 | 0.00009 |
| 8 | 0.350 | 0.007 | 0.16528 | 0.0007 | 0.00503 | 0.00010 |
| 9 | 0.400 | 0.008 | 0.18652 | 0.0007 | 0.00575 | 0.00011 |
| 10 | 0.450 | 0.009 | 0.20982 | 0.0007 | 0.00646 | 0.00013 |
| 11 | 0.500 | 0.010 | 0.2334 | 0.0007 | 0.00718 | 0.00014 |

Table 12. Calculated uncertainty for IM, U and B

A.3. Uncertainty for magnetic field distribution inside the solenoid

From the previous part, we know that KH=32.2±0.6 V/T. Using the first measurement as an example, we can calculate that

uU=10.91×10-3×0.05%+6×10-4=0.0006 V

Since B=UKH, we can calculate that

uB =∂B∂U∙uU2+∂B∂KH∙uKH2=uUKH2+UKH2∙uKH2=0.000632.22+10.91×10-332.22∙0.62=0.00002 T

The table below (Table 13) lists the uncertainty calculated for rest data.

|  | x [cm] | ux [cm] | U V | uU [V] | B [T] | uB [T] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.00 | 0.05 | 0.01091 | 0.0006 | 0.00034 | 0.00002 |
| 2 | 0.50 | 0.05 | 0.01404 | 0.0006 | 0.00044 | 0.00002 |
| 3 | 1.00 | 0.05 | 0.01927 | 0.0006 | 0.00060 | 0.00002 |
| 4 | 1.50 | 0.05 | 0.02675 | 0.0006 | 0.00083 | 0.00002 |
| 5 | 2.00 | 0.05 | 0.03860 | 0.0006 | 0.00120 | 0.00003 |
| 6 | 2.50 | 0.05 | 0.05368 | 0.0006 | 0.00167 | 0.00004 |
| 7 | 3.00 | 0.05 | 0.07097 | 0.0006 | 0.00220 | 0.00005 |
| 8 | 3.50 | 0.05 | 0.08549 | 0.0006 | 0.00265 | 0.00005 |
| 9 | 4.00 | 0.05 | 0.09632 | 0.0006 | 0.00299 | 0.00006 |
| 10 | 4.50 | 0.05 | 0.10343 | 0.0007 | 0.00321 | 0.00006 |
| 11 | 5.00 | 0.05 | 0.10781 | 0.0007 | 0.00335 | 0.00007 |
| 12 | 0.25 | 0.05 | 0.01233 | 0.0006 | 0.00038 | 0.00002 |
| 13 | 0.75 | 0.05 | 0.01649 | 0.0006 | 0.00051 | 0.00002 |
| 14 | 1.25 | 0.05 | 0.02274 | 0.0006 | 0.00071 | 0.00002 |
| 15 | 1.75 | 0.05 | 0.03081 | 0.0006 | 0.00096 | 0.00003 |
| 16 | 6.00 | 0.05 | 0.11323 | 0.0007 | 0.00352 | 0.00007 |
| 17 | 7.00 | 0.05 | 0.11577 | 0.0007 | 0.00360 | 0.00007 |
| 18 | 8.00 | 0.05 | 0.11729 | 0.0007 | 0.00364 | 0.00007 |
| 19 | 9.00 | 0.05 | 0.11812 | 0.0007 | 0.00367 | 0.00007 |
| 20 | 10.00 | 0.05 | 0.11880 | 0.0007 | 0.00369 | 0.00007 |
| 21 | 11.00 | 0.05 | 0.11918 | 0.0007 | 0.00370 | 0.00007 |
| 22 | 12.00 | 0.05 | 0.11931 | 0.0007 | 0.00371 | 0.00007 |
| 23 | 13.00 | 0.05 | 0.11943 | 0.0007 | 0.00371 | 0.00007 |
| 24 | 14.00 | 0.05 | 0.11930 | 0.0007 | 0.00370 | 0.00007 |
| 25 | 15.00 | 0.05 | 0.11950 | 0.0007 | 0.00371 | 0.00007 |
| 26 | 16.00 | 0.05 | 0.11971 | 0.0007 | 0.00372 | 0.00007 |
| 27 | 17.00 | 0.05 | 0.11962 | 0.0007 | 0.00371 | 0.00007 |
| 28 | 18.00 | 0.05 | 0.11959 | 0.0007 | 0.00371 | 0.00007 |
| 29 | 19.00 | 0.05 | 0.11931 | 0.0007 | 0.00371 | 0.00007 |
| 30 | 20.00 | 0.05 | 0.11929 | 0.0007 | 0.00370 | 0.00007 |
| 31 | 21.00 | 0.05 | 0.11905 | 0.0007 | 0.00370 | 0.00007 |
| 32 | 22.00 | 0.05 | 0.11836 | 0.0007 | 0.00368 | 0.00007 |
| 33 | 23.00 | 0.05 | 0.11763 | 0.0007 | 0.00365 | 0.00007 |
| 34 | 24.00 | 0.05 | 0.11615 | 0.0007 | 0.00361 | 0.00007 |
| 35 | 25.00 | 0.05 | 0.11410 | 0.0007 | 0.00354 | 0.00007 |
| 36 | 25.50 | 0.05 | 0.11251 | 0.0007 | 0.00349 | 0.00007 |
| 37 | 26.00 | 0.05 | 0.11044 | 0.0007 | 0.00343 | 0.00007 |
| 38 | 26.50 | 0.05 | 0.10680 | 0.0007 | 0.00332 | 0.00007 |
| 39 | 27.00 | 0.05 | 0.10131 | 0.0007 | 0.00315 | 0.00006 |
| 40 | 27.50 | 0.05 | 0.09387 | 0.0006 | 0.00292 | 0.00006 |
| 41 | 27.75 | 0.05 | 0.08843 | 0.0006 | 0.00275 | 0.00005 |
| 42 | 28.00 | 0.05 | 0.08180 | 0.0006 | 0.00254 | 0.00005 |
| 43 | 28.25 | 0.05 | 0.07280 | 0.0006 | 0.00226 | 0.00005 |
| 44 | 28.50 | 0.05 | 0.06613 | 0.0006 | 0.00205 | 0.00004 |
| 45 | 28.75 | 0.05 | 0.05673 | 0.0006 | 0.00176 | 0.00004 |
| 46 | 29.00 | 0.05 | 0.04758 | 0.0006 | 0.00148 | 0.00003 |
| 47 | 29.25 | 0.05 | 0.04091 | 0.0006 | 0.00127 | 0.00003 |
| 48 | 29.50 | 0.05 | 0.03429 | 0.0006 | 0.00106 | 0.00003 |
| 49 | 29.75 | 0.05 | 0.02903 | 0.0006 | 0.00090 | 0.00003 |
| 50 | 30.00 | 0.05 | 0.02411 | 0.0006 | 0.00075 | 0.00002 |
| 51 | 2.25 | 0.05 | 0.04649 | 0.0006 | 0.00144 | 0.00003 |
| 52 | 2.75 | 0.05 | 0.06237 | 0.0006 | 0.00194 | 0.00004 |

Table 13. Calculated uncertainty for x, U and B

Since Bx and IM_t are theoretical value, we can assume that uBx=0 and uIM_t=0. For IM_e=250 mA, according to the precision or uncertainty of the device, we can calculate its uncertainty as follows

uIM_e=250×2%=5 mA=0.005 A

Then, according to the B=Bx×IM_eIM_t, we can calculate the uncertainty for B as

uB=∂B∂Bx=0∙uBx=02+∂B∂IM_t∙uIM_t2+∂B∂IM_e∙uIM_e2=BxIM_t∙uIM_e2=Bx0.1∙0.0052=0.05Bx=0.05×1.4366×10-3=0.00007 T

The whole table is listed below (Table 14)

| x [cm] | Bx [mT] | B [T] | uB [T] |
| --- | --- | --- | --- |
| 0.0 | 0.7233 | 0.00181 | 0.00004 |
| 0.5 | 0.9261 | 0.00232 | 0.00005 |
| 1.0 | 1.0863 | 0.00272 | 0.00005 |
| 1.5 | 1.1963 | 0.00299 | 0.00006 |
| 2.0 | 1.2685 | 0.00317 | 0.00006 |
| 3.0 | 1.3478 | 0.00337 | 0.00007 |
| 4.0 | 1.3856 | 0.00346 | 0.00007 |
| 5.0 | 1.4057 | 0.00351 | 0.00007 |
| 6.0 | 1.4173 | 0.00354 | 0.00007 |
| 7.0 | 1.4245 | 0.00356 | 0.00007 |
| 8.0 | 1.4292 | 0.00357 | 0.00007 |
| 9.0 | 1.4323 | 0.00358 | 0.00007 |
| 10.0 | 1.4343 | 0.00359 | 0.00007 |
| 11.0 | 1.4356 | 0.00359 | 0.00007 |
| 12.0 | 1.4363 | 0.00359 | 0.00007 |
| 13.0 | 1.4366 | 0.00359 | 0.00007 |
| 14.0 | 1.4363 | 0.00359 | 0.00007 |
| 15.0 | 1.4356 | 0.00359 | 0.00007 |
| 16.0 | 1.4343 | 0.00359 | 0.00007 |
| 17.0 | 1.4323 | 0.00358 | 0.00007 |
| 18.0 | 1.4292 | 0.00357 | 0.00007 |
| 19.0 | 1.4245 | 0.00356 | 0.00007 |
| 20.0 | 1.4173 | 0.00354 | 0.00007 |
| 21.0 | 1.4057 | 0.00351 | 0.00007 |
| 22.0 | 1.3856 | 0.00346 | 0.00007 |
| 23.0 | 1.3478 | 0.00337 | 0.00007 |
| 24.0 | 1.2685 | 0.00317 | 0.00006 |
| 24.5 | 1.1963 | 0.00299 | 0.00006 |
| 25.0 | 1.0863 | 0.00272 | 0.00005 |
| 25.5 | 0.9261 | 0.00232 | 0.00005 |
| 26.0 | 0.7233 | 0.00181 | 0.00004 |

Table 14. Calculated uncertainty for B